1. Define domestic sewage and stormwater.

- Domestic sewage: Wastewater generated from households due to activities like bathing, washing, cooking, and toilet use.

- Stormwater: Water originating from rainfall runoff collected from streets, roofs, and open areas.

2. What is meant by sewage flow variation?

Sewage flow variation refers to fluctuations in sewage discharge over time due to changes in human activity, such as peak flow in morning and evening and lower flow at night.

3. State two common shapes of sewer pipes.

- Circular sewers

- Egg-shaped sewers

4. What is the purpose of a manhole in sewerage systems?

A manhole provides access to underground sewers for inspection, cleaning, and maintenance.

5. Define sullage.

Sullage is wastewater generated from household washing activities such as kitchen cleaning and bathing, but excluding toilet waste.

6. What are small bore sewer systems?

Small bore sewers are a type of simplified sewerage system that conveys only settled sewage, where solids are removed at household septic tanks before entering the sewer.

7. Name any two problems caused by improper disposal of sewage.

- Water pollution and spread of waterborne diseases

- Environmental degradation and foul odor

8. What is the function of a sewage pumping station?

A sewage pumping station lifts wastewater from low-lying areas to higher elevations when gravity flow is not possible.

9. Define aerobic wastewater treatment.

Aerobic treatment is a biological process where microorganisms break down organic matter in the presence of oxygen.

10. Give two examples of suspended growth systems.

- Activated Sludge Process

- Aerated Lagoon

✅ 1. Differentiate between domestic sewage and stormwater.

| Parameter | Domestic Sewage | Stormwater |
| --- | --- | --- |
| Source | Generated from households and commercial activities | Result of rainfall runoff |
| Composition | Contains organic matter, detergents, fecal matter, nutrients, grease | Contains soil particles, debris, and sometimes oil and suspended solids |
| Pollution Strength | High BOD and COD | Usually low pollution level but may pick up contaminants from urban surfaces |
| Treatment Required | Requires full biological treatment before disposal | May require minimal treatment unless contaminated |
| Flow Pattern | Relatively constant with daily variations | Highly variable and depends on rainfall intensity |

Domestic sewage requires detailed treatment due to strong organic contamination, whereas stormwater often needs only sedimentation or filtration unless heavily polluted.

✅ 2. Explain the factors affecting the quantity of sewage generated in an urban area.

Factors affecting sewage quantity include:

- Population and Population Growth:
Higher population results in greater sewage generation.

- Water Supply per Capita:
More water consumption leads to more wastewater generation, typically 70–90% of water supplied becomes sewage.

- Industrial and Commercial Activities:
Presence of industries increases wastewater discharge dramatically.

- Living Standards and Lifestyle:
Areas with washing machines, flushing systems, and better facilities produce more wastewater.

- Infiltration and Inflow:
Groundwater infiltration and stormwater entering defective sewers may increase sewage volume.

- Seasonal Changes:
Seasonal variations like tourist seasons and festivals temporarily increase sewage flow.

✅ 3. Write short notes on operation and maintenance of sewers.

Regular operation and maintenance (O&M) of sewers ensures efficient sewage conveyance and prevents system failure. Key tasks include:

- Routine Inspection:
Checking sewer lines, joints, manholes, and pumping stations for damage.

- Cleaning Operations:
Removal of grit, grease, and blockages using rodding, flushing, jetting, or mechanical cleaning.

- Repair of Damages:
Fixing cracks, joint leaks, and replacing broken pipes to prevent infiltration or exfiltration.

- Maintaining Records:
Logs of inspection, maintenance, and cleaning activities help future planning and scheduling.

- Monitoring Pumping Stations:
Ensuring pumps and electrical controls work efficiently.

Proper O&M prevents flooding, odor issues, pipe collapse, and public health hazards.

✅ 4. Discuss various shapes of sewer sections with their advantages.

Common sewer shapes include:

1. Circular Section

- Most common type.

- Easy to manufacture and install.

- Strong under external load.

- Efficient hydraulic flow at medium to high discharge.

2. Egg-Shaped Section

- Narrow bottom and wide top.

- Good for areas with high flow variation, ensuring higher velocity during low flow.

- Minimizes sediment deposition.

3. Horseshoe Section

- Used for large sewers and tunnels.

- Suitable for large discharge and easy access for maintenance.

4. Rectangular Section

- Used in open channels and large underground conduits.

- Simple to construct but less stable under load.

The choice depends on flow characteristics, construction cost, and structural stability.

✅ 5. What are sewer appurtenances? Explain any three.

Sewer appurtenances are auxiliary structures provided in the sewerage system to assist flow regulation, inspection, cleaning, and maintenance.

Examples:

a) Manholes

Vertical shafts allowing access for inspection, cleaning, and ventilation of sewers.

b) Drop Manholes

Used when two sewer lines meet at different elevations; prevents erosion due to high-velocity discharge.

c) Catch Basins

Structures that trap grit, leaves, and debris from stormwater before entering sewers.

d) Inverted Siphons

Used to carry sewage under obstacles like rivers or railway tracks.

These structures ensure smooth functioning and durability of sewer systems.

✅ 6. Describe the principles and applications of small bore sewer systems.

Principle

Small bore sewer systems convey only settled sewage, meaning solids are removed at septic tanks or interceptor tanks before entering the sewer.

Features

- Smaller pipe diameter

- Lower construction cost

- Reduced excavation depth

Applications

- Low-income or developing urban areas

- Rural communities

- Areas with difficult soil conditions

- Where conventional sewerage is costly

Advantages

- Less maintenance

- Minimal blockages

- Lower wastewater handling volume at treatment plants

Small bore systems are economical and effective alternatives to conventional sewerage.

✅ 7. Explain the pollution problems caused by improper sewage disposal.

Improper sewage disposal causes several environmental and health problems:

- Water Pollution:
Untreated sewage contaminates rivers, lakes, and groundwater with pathogens, nutrients, and toxins.

- Spread of Diseases:
Waterborne diseases like cholera, dysentery, and typhoid increase in communities.

- Eutrophication:
Excess nutrients lead to algal bloom and depletion of dissolved oxygen.

- Soil Pollution:
Harmful chemicals degrade soil fertility and crop production.

- Odor and Public Nuisance:
Decomposing sewage generates foul gases and affects living conditions.

Proper treatment and safe disposal are essential to protect public health and the environment.

✅ 8. Write short notes on National River Cleaning Plans in India.

The Government of India has launched several programs to restore polluted rivers:

1. Ganga Action Plan (GAP)

- Launched in 1985.

- Focused on sewage collection, treatment, and preventing direct discharge into Ganga.

2. National River Conservation Plan (NRCP)

- Extended cleaning efforts to other rivers like Yamuna, Godavari, and Krishna.

- Focus on reducing pollution load and improving water quality.

3. Namami Gange Program

- Launched in 2014.

- Integrates sewerage creation, river surface cleaning, industrial effluent control, and public awareness.

These programs aim to restore river ecosystems, improve water quality, and reduce pollution.

✅ 9. Compare aerobic and anaerobic wastewater treatment systems.

| Feature | Aerobic Treatment | Anaerobic Treatment |
| --- | --- | --- |
| Oxygen Requirement | Requires oxygen | No oxygen required |
| Microorganisms | Aerobic bacteria | Anaerobic bacteria |
| Sludge Production | Produces more sludge | Produces lesser sludge |
| Energy Requirement | High (for aeration) | Low |
| By-products | CO₂ and biomass | Methane and biogas |
| Examples | Activated Sludge Process, Trickling Filter | UASB reactor, Septic tank |

Aerobic systems provide faster treatment and better effluent quality, while anaerobic systems are more energy-efficient and suitable for high-strength wastewater.

✅ 10. Discuss the quality requirements for recycling treated wastewater for industrial and agricultural use.

A. Industrial Use

- Low suspended solids and turbidity to prevent pipe scaling.

- Low hardness for boiler feed water.

- Low organic content (BOD, COD).

- Chlorides and silica must be within acceptable limits for cooling systems.

B. Agricultural Use

- Acceptable salinity (EC) and sodium adsorption ratio (SAR).

- Low concentration of heavy metals to avoid soil toxicity.

- Pathogens and fecal coliforms should meet WHO reuse guidelines.

General Requirements

- Adequate removal of nutrients, pathogens, and suspended solids.

- No foul odor or color.

- Continuous monitoring to ensure environmental and crop safety.

Recycling wastewater reduces freshwater demand and supports sustainability in water management.

1. Explain in detail the design considerations of a sewerage system.

Design of a sewerage system involves collecting, transporting and disposing wastewater in a sanitary manner. The key design considerations include:

1. Population Forecast

Future population is estimated for 30 years using methods like:

- Arithmetic Increase

- Geometric Increase

- Incremental Increase
This determines future sewage quantity.

2. Quantity of Sewage

Sewage flow includes:

- Domestic wastewater

- Industrial wastewater

- Infiltration/inflow
A typical per capita sewage flow ranges from 70–120 LPCD.

3. Peak Factor

Sewage flow varies over the day. Peak discharge is calculated using formulas like:

- Harmon’s formula

- Babbit’s formula

4. Sewer Layout and Alignment

Sewer lines follow:

- Road alignments

- Minimum disturbance to utilities

- Gravity flow wherever possible

5. Sewer Material

Materials selected based on durability and cost:

- RCC

- Cast iron

- UPVC

- Vitrified clay

6. Hydraulic Design

Sewers must:

- Flow partially full

- Maintain self-cleansing velocity (0.6–0.9 m/s)

- Avoid excessive velocities (<3 m/s)

7. Appurtenances

Manholes, drop connections, clean-outs and inspection chambers must be regularly located.

8. Pumping Requirements

If gravity flow is not possible, pump stations are included.

9. Maintenance Provisions

Sewers should be accessible for desilting and repair.

10. Environmental and Public Health Safety

The design must ensure:

- No leakage

- No groundwater pollution

- Odor-free operation

2. Discuss National River Cleaning Plans and their significance.

The National River Cleaning Plans aim to restore the water quality and ecological health of polluted rivers in India.

Major Programs

1. Ganga Action Plan (GAP) – 1985

- First national program for cleaning the Ganga.

- Included STP construction, low-cost sanitation, crematoriums, and sewer rehabilitations.

2. National River Conservation Plan (NRCP) – 1995

- Expanded river cleaning to 34 major rivers across 160 towns.

- Included treatment of industrial effluents, solid waste, and sewage interception.

3. Namami Gange Mission – 2014

- A flagship Rs. 20,000-crore program.

- Focus areas:

  - Wastewater treatment

  - River surface cleaning

  - Biodiversity conservation

  - Ganga Grams (rural sanitation)

  - Public participation

Key Components of River Cleaning

- Construction of STPs

- Interception and diversion of raw sewage

- Monitoring water quality

- Riverbank cleaning

- Industrial wastewater regulation

Significance

- Reduced pollution discharge into rivers.

- Improved river water quality.

- Protected aquatic ecosystems.

- Enhanced recreational and tourism value.

- Greater awareness and public participation.

3. Explain aerobic and anaerobic treatment systems with examples.

Wastewater treatment can use biological processes that depend on the presence or absence of oxygen.

A. Aerobic Treatment Systems

Microorganisms degrade organic matter in the presence of oxygen.

Mechanism

Oxygen supports bacteria that convert:

- Organic matter → CO₂ + water + new cells

Common Aerobic Systems

- Activated Sludge Process

  - Aeration tank + secondary clarifier.

  - Widely used in municipal STPs.

- Trickling Filters

  - Wastewater trickles through a medium supporting attached microbes.

- Oxidation Ditches

  - Continuous loop channels with mechanical aeration.

Advantages

- High efficiency for BOD removal (85–95%)

- Less odor

Disadvantages

- Higher energy requirement (aeration)

B. Anaerobic Treatment Systems

Microorganisms break down organic matter without oxygen.

Mechanism

Organic matter → Methane + CO₂ + sludge

Common Anaerobic Systems

- UASB (Upflow Anaerobic Sludge Blanket)

  - Widely used in India.

  - Generates methane that can be used as fuel.

- Anaerobic Lagoons

  - Suitable for rural and industrial wastewater.

- Septic Tanks

  - Used for small households and institutional complexes.

Advantages

- Low operational cost

- Energy recovery from methane

Disadvantages

- Slower treatment

- Possible odor issues

Conclusion

| Parameter | Aerobic | Anaerobic |
| --- | --- | --- |
| Oxygen | Required | Not required |
| Energy Use | High | Low |
| Sludge | More | Less |
| Gas Output | None | Biogas (CH₄) |
| Treatment Rate | Faster | Slower |

Both are selected based on wastewater characteristics and economic considerations.

4. Describe suspended and attached growth processes with suitable diagrams and examples.

Biological wastewater treatment systems are categorized based on how microorganisms are supported.

A. Suspended Growth Systems

Biomass is freely suspended in the wastewater.

Examples

- Activated Sludge Process (ASP)

  - Aeration tank suspends microorganisms.

  - Treated in clarifier for solid–liquid separation.

- Oxidation Ditch

  - Extended aeration in a loop channel.

- Aerated Lagoons

  - Large basins with mechanical aeration.

Advantages

- High BOD removal efficiency

- Good process control

Disadvantages

- Requires continuous aeration

- Higher sludge production

B. Attached Growth Systems

Microorganisms grow as a biofilm on a solid medium.

Examples

- Trickling Filter

  - Stones or plastic media act as biofilm support.

- Rotating Biological Contractor (RBC)

  - Discs rotate through wastewater allowing biomass growth.

- Submerged Aerated Fixed Film Reactor

Advantages

- Less sludge production

- Lower power consumption

Disadvantages

- Possible media clogging

- Less flexible for shock loading

Comparison

| Feature | Suspended Growth | Attached Growth |
| --- | --- | --- |
| Biomass Form | Free suspension | Film on surfaces |
| Examples | ASP, oxidation ditch | Trickling filter, RBC |
| Sludge Volume | High | Low |
| Oxygen need | High | Low–medium |

5. Explain the importance and process of recycling sewage and state the quality requirements for various uses.

Recycling of treated sewage is essential due to increasing water scarcity.

1. Importance

- Reduces load on freshwater sources

- Reduces discharge into water bodies

- Saves treatment and pumping costs

- Improves environmental health

- Supports sustainable development

2. Sources for Reuse

- Treated municipal sewage

- Partially treated industrial effluents

3. Process of Sewage Recycling

- Collection

  - Raw sewage collected through sewerage network.

- Primary Treatment

  - Removal of settleable solids via sedimentation.

- Secondary Treatment

  - Biological treatment (ASP, UASB, trickling filters)

- Tertiary/Advanced Treatment

  - Filtration

  - Disinfection (chlorine/UV/ozone)

  - Membrane processes (UF/RO)

- Distribution

  - Pumping sewage back to industrial, irrigation or domestic reuse network.

4. Quality Requirements for Various Reuse Applications

| Use | Design Standards |
| --- | --- |
| Irrigation | BOD < 100 mg/L, Pathogen removal, No toxic chemicals |
| Industrial Cooling | SS < 10 mg/L, low hardness and TDS |
| Flushing/Urban Reuse | BOD < 10 mg/L, complete disinfection |
| Groundwater Recharge | Removal of pathogens and nitrates |
| Agriculture | Nutrients (N, P, K) beneficial but heavy metals should be low |

5. Examples of Successful Recycling

- Chennai industrial reuse schemes

- Bangalore city lake rejuvenation projects

Conclusion

Sewage recycling enhances water security, supports agriculture and industry, and protects the environment.