LED Blinking Using 8051 Microcontroller Trainer Kit

Part A: Code Development in Keil µVision

Project Setup

Step 1: Create New Project

- Open Keil µVision IDE[1][2]

- Navigate to Project → New µVision Project

- Select project location and provide project name

- Click Save

Step 2: Select Target Device

- In the device selection window, choose Atmel AT89C51 microcontroller[2][3]

- Click OK to confirm

- When prompted to copy STARTUP.A51 file, select No (startup file not required)[2][4]

Code Development

Step 3: Create Source File

- Go to File → New to create a new file

- Write your LED blinking code

- Save the file with .c extension for C language (e.g., led_blink.c)[2]

- For assembly language programs, use .a extension

Step 4: Add File to Project

- Right-click on Source Group 1 in the project tree

- Select Add Files to Group

- Browse and select the saved source file

- Click Add to include it in the project[2]

Project Configuration

Step 5: Configure Target Options

- Navigate to Project → Options for Target

- In the Target tab, locate Xtal (MHz) field

- Change the crystal oscillator frequency to 11.0592 MHz[5][6][7]

- Go to Output tab

- Enable the checkbox Create HEX File[2][8]

Step 6: Build Project

- Select Project → Build Target (or press F7)[2]

- Check the Build Output window for compilation logs

- Verify message: "creating hex file from led_blink"

- Navigate to project folder to confirm .hex file generation (contains low-level code in hexadecimal format)[2]

Part B: Programming Microcontroller Using Flash Magic

Hardware Preparation

Step 7: Configure Trainer Kit

- Shift the jumper to left side (Load mode) on the trainer kit[9][10]

- Power on the trainer kit

- Connect the kit to PC using USB cable

Step 8: Identify COM Port

- Open Device Manager on PC

- Expand Ports (COM & LPT) section

- Note the COM port number for USB Serial Port[10]

Flash Magic Configuration

Step 9: Software Settings

- Launch Flash Magic application[9][10]

- Configure the following parameters in sequence:

  - COM Port: Select the identified COM port from Step 8

  - Baud Rate: Set to 9600 (default)[10]

  - Device: Select 89C51RD2 (or 89C51RD2H_EX)[11][10]

  - Oscillator Frequency: Enter 11.0592 MHz[5][7]

Step 10: Program Options

- Enable checkbox: Verify after programming[10]

- Enable checkbox: Execute after programming

- Enable checkbox: Erase blocks used by Hex File[10]

Step 11: Load and Program

- Click Browse button in the Hex File section

- Navigate to project folder and select the generated .hex file[10]

- Click Start button to begin programming

- Monitor the status bar showing:

  - Erasing blocks (removing old memory contents)[9]

  - Programming flash memory

  - Verifying programmed data

  - Finished message upon completion[9][10]

Hardware Configuration and Execution

Step 12: LED Connection

- If the code uses Port 1 (P1), select Port 1 on the trainer kit

- Connect jumpers from the selected port pins to the LED section on the board

Step 13: Execute Program

- Shift the jumper from Load mode to Run mode[9]

- Press the Reset button to execute the program[10]

- Observe the LED blinking as per the programmed pattern

Code Modification (Optional)

Step 14: Editing and Reprogramming

- To modify the code, open the source file in Keil µVision

- Make necessary changes

- Click the Build icon (small hammer icon) in the ribbon/toolbar[2]

- Repeat Flash Magic programming steps (Steps 9-13) to upload the updated code