ASSIGNMENT – 2

POWER SYSTEM PROTECTION

Group - A

- Draw the protection scheme to detect a turn to turn fault in a winding of an alternator.

- State and draw the  protection of Bus bar.

- Mention the differences between current transformers used for protection and measurement.

- Why secondary of CT should not be open circuited?

- List the abnormal conditions in a transformer against the protection is necessary.

- List the various schemes of protection for feeders.

- A three phase transformer having line voltage ratio 440V/11KV connected in Star/Delta. The protection transformer on LV side has a current ratio of 500/5.  What must be the ratio of the protection transformer on HV side?

- What is burden of CT?  How is it specified?

- Why secondary of CT should not be open circuited?

- What do you mean by ‘differential protection’? Why is differential protection

called ‘unit-protection’?

- What are the disadvantages of the simple differential protection scheme?

- Why IDMT relays are widely used for overcurrent protection.

Group – B

- Explain in details the  Merz-Price circulating current protection of an alternator.

- Identify and explain the different protection schemes necessary for the protection of  power transformer.

- Write short notes on       (i) Transley relay

(ii) Phase comparison Career Current Protection scheme

- A three phase 66kv/11kv transformer is connected as star/delta. The transformer is protected by Mertz-Price circulating current system. Protecting  current transformers on the low voltage side have a ratio of 250/5. Find the ratio of the current transformer on high voltage side.

- Explain stepped time-distance characteristics of three distance relaying units used for I, II and III zone of protection.

- Explain impedance relay characteristic on the R-X diagram. Discuss the range setting of three impedance relays placed at a particular location. Discuss why the I zone unit is not set for the protection of 100% of the line.